# From:  Andrew Zulu (Southern DataStream)

# To:      Michael Hanson (Boyle Engineering)

# Status Report [ 08/27/01 ]

The XP-SWMM results along the Gator Slough shows a major difference with those of Johnson’s HEC simulation in the following parameters:

- Water surface elevations

- Weir Height-discharge relationship

The water surface elevations along the Gator Slough are shown in Figure 1. It is evident that the results match up well for the 1-year calibration but not quite as well for the 25-year 3-day design event except around the inflow nodes and the mouth of the Slough. A water balance for the calibration and the 1-year, 5-year, 25-year and 100-year design events (Table 1) shows that the inflow from Hwy 41 gives the discontinuity in the overall balance. It is not immediately clear why this is so as the output file in XP-SWMM also excludes inflow information in the continuity check for surface water. XP-SWMM however gives ‘inflow to node’ figures aside of the continuity check, these figures have been used as indicated in Table 1.

Current investigations involve establishing the cause of this disparity. Emphasis has been laid on the inflow from Hwy 41 gauge station. A second approach has been to investigate any tidal or other boundary conditions assumed by Johnson.

It also seems that it may be just a difference in assumed head (hence discharge) of the two simulations because of using different rainfall depths. Unfortunately Johnson does not give information on all its inputs for the simulation more specifically there is no information on rainfall depth, infiltration, evapo-transpiration and runoff.

# Table 1. Water budget

# To verify the results the other approach being tried is computing the head-discharge relationships for the 4 weirs (#4, 9, 11 and 19) in the Gator Slough. Tables 2,3,4 and 5 give information on Weirs #4, #9, #11 and #19 respectively giving the reported discharge and the discharge obtained using the weir equation. The weir relationships used for #4, #9 and #11 is consistent with Metcalf and Eddy report but the one for Weir #19 is not yet verified correct with other sources.

## Weir #4

Length, L = 230 ft

Spill crest elevation = 6.3 ft

Assumed: Q = 3.33*L*H1.5

Table 2. Output summary for Weir #4

| Event | WSEL |  | Head, H |  | Disch, Q |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | XPSWMM | J-Report | XPSWMM | J-Report | XPSWMM | Weir eqn [XP] | J-Report | Weir eqn [JR] |
| 1-year | 6.81 | 6.69 | 0.51 | 0.39 | 619 | 279 | 190 | 187 |
| 5-year | 7.1 | 7.4 | 0.8 | 1.1 | 824 | 548 | 955 | 884 |
| 25-year | 7.23 | 12.24 | 0.93 | 5.94 | 1103 | 687 | 2940 | 11088 |

Profile: Reinforced concrete, zigzag weir. This is a polygonal weir resembling interconnected boxes, no notch, no gates.

Figure 3. Weir #4 Plan (top) view [not to scale]

Flow

## Weir #9

Length, L = 220 ft

Spill crest elevation = 8.5 ft

Assumed: Q = 3.33*L*H1.5

Table 3. Output summary for Weir #9

| Event | WSEL |  | Head, H |  | Disch, Q |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | XPSWMM | J-Report | XPSWMM | J-Report | XPSWMM | Weir eqn [XP] | J-Report | Weir eqn [JR] |
| 1-year | 8.96 | 8.86 | 0.46 | 0.36 | 667 | 229 | 180 | 158 |
| 5-year | 9.53 | 11.34 | 1.03 | 2.84 | 1055 | 766 | 870 | 3506 |
| 25-year | 10.31 | 16.33 | 1.81 | 7.83 | 813 | 1784 | 2695 | 16051 |

Profile: Reinforced concrete weir with shape resembling a set of stairs across the canal., no notch, no gates.

Flow 







Figure 5. Weir #9 Side view [not to scale]

Downstream elevation

Upstream elevation

## Weir #11

Length, L = 175 ft

Spill crest elevation = 2.4 ft

Assumed: Q = 3.33*L*H1.5

Table 4. Output summary for Weir #11

| Event | WSEL |  | Head, H |  | Disch, Q |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | XPSWMM | J-Report | XPSWMM | J-Report | XPSWMM | Weir eqn [XP] | J-Report | Weir eqn [JR] |
| 1-year | 3.02 | 2.94 | 0.62 | 0.54 | 404 | 284 | 220 | 231 |
| 5-year | 3.72 | 3.96 | 1.32 | 1.56 | 766 | 884 | 1140 | 1135 |
| 25-year | 4.09 | 5.64 | 1.69 | 3.24 | 1589 | 1280 | 3500 | 3399 |

Profile: Reinforced concrete weir, no notch, one slide gate.

Figure 7. Weir #11

## Weir #19

Length, L = 92 ft

Spill crest elevation = 10.1 ft

Assumed: Q = 3.3*L*H1.58

Table 5. Output summary for Weir #19

| Event | WSEL |  | Head, H |  | Disch, Q |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | XPSWMM | J-Report | XPSWMM | J-Report | XPSWMM | Weir eqn [XP] | J-Report | Weir eqn [JR] |
| 1-year | 11.31 | 10.57 | 1.21 | 0.47 | 402 | 410 | 140 | 92 |
| 5-year | 14.44 | 14.25 | 4.34 | 4.15 | 478 | 3087 | 600 | 2876 |
| 25-year | 15.65 | 18.83 | 5.55 | 8.73 | 281 | 4553 | 1880 | 9313 |

Profile: 92’ Long weir located diagonally inside two box culverts.

Figure 9. Weir #19

Figures 2,4,6 and 8 give the plotted relationship as given by Tables 2,3,4 and 5 respectively.

It is evident that only Weir #11 shows results agreeing closely with theoretical weir equation calculations.

### Summary

The remaining problems to be solved are summarized as follows:

- Addressing the water surface elevations disparity with Johnson

- Assessing the flood impact of the 3 design events (streets, septic tank, etc)

- Determining the water retention quantities of the 3 design events

- Addressing the Head-discharge relationship for weirs to verify our model

- Final report (should not take along if all above are addressed!)

The author is looking into other approaches of attacking the problems and hopes that there will be a solution in the next few days.